Supplementary Information

S1. Numerical sensitivity of the finite-element discretisation and front extraction

Numerical sensitivity was assessed to verify that the extracted observables were not controlled by the finite-element mesh, time step, or front-sampling resolution. The results are reported in Table S5. These tests quantify differences between numerical simulations and should not be interpreted as errors relative to experimental measurements. For the mesh-sensitivity study, progressively refined finite-element meshes were compared against a reference case with Nr=240, Nz=544, and Δt=0.25 s. For the time-step study, simulations were compared against a reference case using Nr=180, Nz=408, and Δt=0.125 s. For the front-sampling study, the finite-element solution was kept fixed and only the post-processing sampling resolution used to extract the front was varied. The finest sampling case, Nz,front=1200, Nr,front=35, and Nz,front,curve=600, was used as the sampling reference.

Differences reported in Table S5 were computed after interpolating the compared histories onto common time points. Probe-temperature differences were calculated from the three simulated probe-temperature histories. Front-position differences were calculated from the centreline and near-wall trajectories, zf(t) and zf,w(t). Curved-front differences were calculated from zf(r,t) after interpolation in time and radius. Front-passage-time differences were calculated at z=3.0, 6.2, and 11.0 mm, with Table S5 reporting the maximum absolute difference across these heights. Freezing-completion-time differences were calculated relative to the corresponding reference case. Based on this analysis, Nr=180, Nz=408, Δt=0.25 s, Nz,front=800, Nr,front=25, and Nz,front,curve=400 were used for the simulations reported in the main text.

S2: First-order cryostage response parameters

The plate-temperature response parameters used in the open-loop trajectory evaluations were obtained from the cryostage characterization telemetry, rather than adjusted during the trajectory optimization.

Table S1: Linear calibration coefficients implemented in the firmware for the cold-plate RTD and thermocouple channels.

| Sensor channel | Description | a | b |
| --- | --- | --- | --- |
| Cold-plate RTD | Aluminium cold plate | 0.90096545 | 0.23752064 |
| Thermocouple 1 | In-sample, z = 3.0 mm | 0.93444306 | 1.07136786 |
| Thermocouple 2 | In-sample, z = 6.2 mm | 0.93842745 | 1.01716721 |
| Thermocouple 3 | In-sample, z = 11.0 mm | 0.95224917 | 0.68714428 |
| Thermocouple 4 | External mould-side thermocouple | 0.945864911 | 0.8695248 |

.

Table S2: Effective local surrounding-air temperatures used in the convective boundary condition.

| Plate temperature, Tplate (°C) | Effective T∞ (°C) |
| --- | --- |
| −10 | 9.708 |
| −15 | 7.568 |
| −20 | 5.75 |

Note: T∞ values were calculated from the external mould-side thermocouple and used to define T∞(Tplate) by linear interpolation, with nearest-value clamping outside the measured range.

Table S3: Constant material properties and fixed density assignment used in the model.

| Region / material | Treatment in the model | ρ (kg m⁻³) | cₚ (J kg⁻¹ K⁻¹) | k (W m⁻¹ K⁻¹) | Note |
| --- | --- | --- | --- | --- | --- |
| PLA mould | Constant properties | 1250 | 1800 | 0.13 | Assigned to the mould region. |
| Air headspace | Constant properties | 1.2 | 1005 | 0.026 | Assigned to the headspace and pre-fill cavity. |

Table S4: Tabulated liquid-water and ice thermophysical properties used for interpolation.

| Phase | T (°C) | ρ (kg m⁻³) | cₚ (J kg⁻¹ K⁻¹) | k (W m⁻¹ K⁻¹) |
| --- | --- | --- | --- | --- |
| Ice | -40.0 | 920.8 | 1818 | 2.63 |
| Ice | -35.0 | 920.4 | 1851 | 2.57 |
| Ice | -30.0 | 920 | 1882 | 2.50 |
| Ice | -25.0 | 919.6 | 1913 | 2.45 |
| Ice | -20.0 | 919.4 | 1943 | 2.39 |
| Ice | -15.0 | 919.4 | 1972 | 2.34 |
| Ice | -10.0 | 918.9 | 2000 | 2.30 |
| Ice | -5.0 | 917.5 | 2027 | 2.25 |
| Ice | 0.0 | 916.2 | 2050 | 2.22 |
| Liquid water | 0.0 | 1000 | 4217 | 0.566 |
| Liquid water | 5.0 | 1000 | 4204 | 0.571 |
| Liquid water | 10.0 | 1000 | 4193 | 0.580 |
| Liquid water | 15.0 | 1000 | 4185 | 0.588 |
| Liquid water | 20.0 | 1000 | 4182 | 0.598 |
| Liquid water | 25.0 | 1000 | 4181 | 0.606 |

Note. Liquid-water density was fixed at 1000 kg m⁻³ for all tabulated liquid temperatures. For temperatures below Tf, ice density, specific heat capacity, and thermal conductivity were interpolated from the ice table. At T = Tf = 0 °C, the implementation assigns liquid-water base properties; the ice values at 0 °C are included only as the upper interpolation bound for the ice table.

Source note. Values are those implemented in the simulation materials file. The corresponding literature source cited in the manuscript is Incropera et al. [6].

Table S5: Numerical sensitivity of the extracted simulation observables.

| Test | Case | Reference | Nr | Nz | dt (s) | Nz,front | Nr,front | Nz,front, curve | eT,RMS (ºC) | ezf,RMS (mm) | ezfw,RMS (mm) | ezf(r),RMS (mm) | Delta tfreeze (s) | max ¦Delta tpassage¦ (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mesh | coarse | mesh ref. | 90 | 204 | 0.25 | 800 | 25 | 400 | 0.051 | 0.045 | 0.034 | N/A | 2.75 | 1.95 |
| Mesh | medium | mesh ref. | 135 | 306 | 0.25 | 800 | 25 | 400 | 0.098 | 0.049 | 0.085 | N/A | -2.00 | 3.72 |
| Mesh | selected | mesh ref. | 180 | 408 | 0.25 | 800 | 25 | 400 | 0.041 | 0.050 | 0.026 | N/A | -1.25 | 2.66 |
| Mesh | reference | mesh ref. | 240 | 544 | 0.25 | 800 | 25 | 400 | 0.0000 | 0.0000 | 0.0000 | N/A | 0.00 | 0.00 |
| Time step | coarse | time ref. | 180 | 408 | 1 | 800 | 25 | 400 | 0.222 | 0.094 | 0.139 | N/A | 10.75 | 6.81 |
| Time step | medium | time ref. | 180 | 408 | 0.5 | 800 | 25 | 400 | 0.043 | 0.037 | 0.042 | N/A | 1.25 | 2.48 |
| Time step | selected | time ref. | 180 | 408 | 0.25 | 800 | 25 | 400 | 0.024 | 0.038 | 0.026 | N/A | -0.75 | 2.81 |
| Time step | reference | time ref. | 180 | 408 | 0.125 | 800 | 25 | 400 | 0.0000 | 0.0000 | 0.0000 | N/A | 0.00 | 0.00 |
| Front sampling | coarse | sampling ref. | 180 | 408 | 0.25 | 400 | 15 | 300 | 0.0000 | 0.0013 | 0.0018 | 0.0023 | 0.00 | 0.29 |
| Front sampling | selected | sampling ref. | 180 | 408 | 0.25 | 800 | 25 | 400 | 0.0000 | 0.0004 | 0.0006 | 0.0014 | 0.00 | 0.02 |
| Front sampling | reference | sampling ref. | 180 | 408 | 0.25 | 1200 | 35 | 600 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.00 | 0.00 |

Note. Values are numerical differences relative to the reference simulation listed in each row, not errors relative to experimental measurements. eT,RMS is the RMS difference across the three probe-temperature histories. ezf,RMS and ezfw,RMS are the RMS differences in the centreline and near-wall front trajectories, respectively. ezf(r), RMS is reported only for the curved-front sampling study. Delta tfreeze is the difference in freezing-completion time relative to the corresponding reference case. max |Delta tpassage| is the maximum absolute front-passage-time difference across z = 3.0, 6.2, and 11.0 mm. Selected cases are shaded yellow, and reference cases are shaded green.